# RGB/WH Control Program – MODE Button and Settings Description

Updated for the new PIR edge-detection and dynamic RGB effects algorithm

Program file: RGB_WH_IR 16F18426 v1.gcb
Inputs: Button 1 — physical input or IR CH1/CH2; Button 2 — physical input or IR CH3/CH4.

# 1. Main Elements

| Element | In code | Meaning |
| --- | --- | --- |
| MODE button | butt_mode = PORTC.5 | Switches the main operating mode. The input has a pull-up; pressing the button applies LOW. |
| Button 1 | butt_1 = PORTA.4 | Main + / ON-OFF / increase button. |
| Button 2 | butt_2 = PORTA.3 | Main - / effects / settings button. A long press switches SET items. |
| PIR sensor | PIR = PORTA.5 | Motion sensor. In the new algorithm, the PIR 0→1 rising edge is important. |
| Photo sensor | AN1 | Detects the light/dark threshold. A higher ADC value means a darker condition. |
| PCB LED | LED_ind = PORTA.0 | In normal mode, indicates the MODE number with 1/2/3 pulses; in SET mode, indicates the SET number with 1/2/3/4 pulses. |
| LED strip | R/G/B/W/WW PWM | Operating lighting, effects and visual indication. |
| Buzzer | PORTC.4 | Confirmation and timer warning sound signals. |

# 2. MODE Button Operation

The MODE button uses inverted logic: released = 1, pressed = 0.

Hold the button until mode_pushtime = 80; with Timer1 at approximately 30.5 interrupts per second, this is about 2.6–3 s.

The mode cycles as follows: MODE 0 → MODE 1 → MODE 2 → MODE 0.

mode_case is stored at EEPROM address 1, so the selected mode is retained after power-off.

After switching, the LED strip indicates the active MODE by color, the PCB LED indicates the mode number by pulse count, and the buzzer gives a confirmation signal.

The MODE button is ignored while SET mode or MODE indication is active.

# 3. Main Operating Modes

## MODE 0 — Outdoor / Canopy Mode

MODE indication: red LED strip flashing; during normal operation the PCB LED gives 1 short pulse per cycle.

The photo sensor enables night mode. When night_mode = 1, the selected RGB background effect and PIR control are active.

PIR is processed on the 0→1 rising edge. On the first edge, max_bright_bit and PIR_bit are activated and the white light fades up gradually.

If the white PIR cycle is already active, the next PIR edge no longer resets brightness to 0; it only refreshes the PIR timer. This prevents the white light from flickering.

While the physical PIR output is 1, max_bright_sec is held at 0. Therefore, the sensor's own internal ON time is not included in the user-set hold time.

When the PIR output changes to 0, max_bright_sec starts counting. If a new 0→1 edge occurs during this time, the timer is reset to 0 again.

When max_bright_sec reaches the time selected in SET 3, the white light gradually fades down to 0; the selected RGB background effect then resumes.

The photo sensor must be positioned or shielded so that the LED strip does not illuminate it directly; otherwise, the lamp's own light may affect night_mode.

## MODE 1 — Room Mode

MODE indication: green LED strip flashing; during normal operation the PCB LED gives 2 short pulses per cycle.

Button 1 switches the lighting ON/OFF with gradual fade-up/fade-down.

Button 2 changes the white-light level; in RGBW effect mode it changes the effect.

PIR is processed on the 0→1 rising edge. PIR may start PIR mode only when dim_sw = 0, rgb_eff_bit = 0 and PIR_bit = 0. If the light or an RGBW effect has already been switched on manually, PIR does not affect that mode.

While PIR = 1, max_bright_sec is reset. When motion ends, the SET 3 hold-time countdown begins.

Static RGBW colors fade up/down gradually through dimm_up_dwn.

## MODE 2 — MODE 1 with Blue Background Mode

MODE indication: blue LED strip flashing; during normal operation the PCB LED gives 3 short pulses per cycle.

Control is the same as in MODE 1.

If the photo sensor detects darkness and no other light/effect is active, a dim blue background remains.

## 3.1. PIR Start of Dynamic RGBW Effects in MODE 1/2

For dynamic RGBW effects 0 and 1, PIR activation no longer jumps immediately to the programmed brightness.

The RGB dynamic-cycle counters (cnt11, cnt, phase, tick, speedSlowCnt) are initialized only on the first PIR activation, when PIR_bit changes from 0 to 1. The blue starting phase is then selected (phase = 4, cnt = 0).

The effect first fades the blue channel from 0 up to bright_lim; RGB_dyn then continues from the blue phase without a color jump. While PIR_bit is already 1, repeated PIR motion no longer resets cnt11, cnt, phase, tick or speedSlowCnt, so the dynamic effect continues uninterrupted.

Effect 0 additionally fades in a small W background up to dutyW = 20; effect 1 operates without the W background.

# 4. Entering Settings Mode

Hold Button 2 for about 2.6–3 s. The program enters settings mode (sett_execute_bit = 1).

Each subsequent long press of Button 2 advances to the next item: SET 1 → SET 2 → SET 3 → SET 4 → exit.

After selecting a SET item, the LED strip briefly indicates the item's color.

In SET mode, the PCB LED indicates the selected SET number by pulse count: SET 1 = 1 pulse, SET 2 = 2, SET 3 = 3, SET 4 = 4.

After the initial indication ends, values are changed with Button 1 and Button 2 according to the selected SET item.

If there is no activity for approximately 30 seconds, the program automatically exits SET mode, displays the MODE indication and gives a beep_set = 20 signal.

# 5. Settings

## SET 1 — Maximum RGB Effect Brightness

Initial LED strip indication: green flashing.

Variable: bright_lim; EEPROM address 4.

Button 1 increases bright_lim by 10; Button 2 decreases it by 10.

At the limits, the W channel is used as a limit indication.

speedBright is recalculated automatically according to bright_lim so that the dynamic RGB effect speed remains more consistent at different brightness levels.

## SET 2 — RGB Effect Speed

Initial LED strip indication: red flashing.

Variable: speedSlow; EEPROM address 3.

Button 1 increases speedSlow — the effect becomes slower.

Button 2 decreases speedSlow — the effect becomes faster.

At the upper/lower limit, the W channel serves as a limit indication.

## SET 3 — PIR-Activated Lighting Hold Time

Initial LED strip indication: blue flashing.

Variables: max_bright_sec_set and max_bright_case_cnt; EEPROM addresses 5 and 8.

Button 1 cycles through fixed times: 50 s → 120 s → 180 s → 240 s → 50 s.

Selection indication: white = 50 s, red = 120 s, green = 180 s, blue = 240 s.

This is the hold time after the physical PIR output changes to 0. While PIR = 1, max_bright_sec is reset.

## SET 4 — Photo Sensor Threshold (Fine Adjustment)

Initial LED strip indication: white flashing.

Variable: lux_set; EEPROM address 2.

Button 1 increases the threshold; Button 2 decreases it.

A hysteresis of ±10 ADC units is used during adjustment.

Indication: white/bright light for one sensor state and dim RGB for the other, allowing the threshold transition to be seen.

At the limits, the W channel serves as a limit indication.

Coarse threshold adjustment is performed with the potentiometer on the PCB.

# 6. LED Indication

## 6.1. LED Strip

| Situation | LED strip indication |
| --- | --- |
| Power-up | Shows the MODE stored in EEPROM: MODE 0 red, MODE 1 green, MODE 2 blue. |
| MODE switching | MODE 0 = red; MODE 1 = green; MODE 2 = blue. |
| SET 1 | Green flashing. |
| SET 2 | Red flashing. |
| SET 3 | Blue flashing. |
| SET 4 | White flashing. |
| SET 3 value | White 50 s; red 120 s; green 180 s; blue 240 s. |
| MODE 1/2 dynamic PIR effect | Gradual start with blue; effect 0 also has a small W background. |

## 6.2. PCB LED

| Situation | PCB LED operation |
| --- | --- |
| Normal MODE 0 | 1 short pulse per cycle. |
| Normal MODE 1 | 2 short pulses per cycle. |
| Normal MODE 2 | 3 short pulses per cycle. |
| SET 1 | 1 slower pulse per SET cycle. |
| SET 2 | 2 slower pulses per SET cycle. |
| SET 3 | 3 slower pulses per SET cycle. |
| SET 4 | 4 slower pulses per SET cycle. |

# 7. Quick Operating Instructions

To change the operating mode, hold the MODE button for about 2.6–3 s.

To enter settings, hold Button 2 for about 2.6–3 s.

Use the next long press of Button 2 to advance to the next SET item.

Change SET 1, SET 2 and SET 4 values with Button 1/Button 2. In SET 3, Button 1 cycles through the fixed times.

After SET 4, hold Button 2 again to exit settings.

If no action is taken for about 30 s in SET mode, the program exits settings automatically.

In MODE 0, if a person remains in the room/area, periodic PIR timer beeps warn that automatic switch-off is approaching; move so that the PIR generates a new edge, or use the manual room mode.

# 8. Settings Stored in EEPROM

| Address | Variable | Meaning |
| --- | --- | --- |
| 1 | mode_case | Active operating mode MODE 0/1/2. |
| 2 | lux_set | Photo sensor threshold. |
| 3 | speedSlow | RGB effect speed. |
| 4 | bright_lim | Maximum RGB effect brightness. |
| 5 | max_bright_sec_set | PIR hold time: 50/120/180/240 s. |
| 6 | main_rgb_eff | MODE 0 RGB background effect number. |
| 7 | main_rgbw_eff | MODE 1/2 RGBW effect number. |
| 8 | max_bright_case_cnt | Selected SET 3 option 0..3. |

# 9. Buzzer

The buzzer is connected to PORTC.4 and is controlled by beep_bit, beep_set and beep.

beep_set = 3 — short confirmation signal for button/effect/lighting actions.

beep_set = 10 — MODE/SET switching signal and operating-timer warning.

beep_set = 20 — automatic SET-mode exit signal and periodic PIR timer warnings.

During an active PIR cycle, warnings occur at max_bright_sec = 40, 110, 170 and 230 s. Therefore, with the 240 s setting the user receives several periodic warnings; with shorter settings only some of these points are reached.

The purpose of the periodic PIR beeps is to warn that the automatic lighting is approaching switch-off; if the person is still in the area, motion creates a new PIR edge and refreshes the time.

In the current code, the operating-timer warning occurs at approximately 2 h 30 min 30 s, and automatic switch-off at approximately 2 h 31 min 40 s.

# 10. Timer Operation and the New PIR Algorithm

## 10.1. Manual Lighting Operating Timer

Uses sec, min and hours.

Counts when sett_execute_bit = 0 and either dim_sw = 1 or rgb_eff_bit = 1 is active.

If both manual states are 0, sec/min/hours are reset.

Button and mode actions reset sec/min/hours in several places, thereby extending the time until automatic switch-off.

In the current code, warn_bit is activated at ~2:30:30; at ~2:31:40 dim_sw and rgb_eff_bit are switched off.

## 10.2. PIR Timer — General Principle

The PIR timer uses max_bright_sec and PIR_bit. min_bright_bit is no longer used in the new algorithm.

Approximately once per second, Timer1 increments max_bright_sec only when PIR_bit = 1; when PIR_bit = 0, the counter is held at 0.

While the physical PIR input is 1, the main logic resets max_bright_sec to 0. Therefore, the hold-time countdown starts only after the PIR output changes to 0.

If PIR produces another 0→1 edge before the timer expires, max_bright_sec is reset and the hold-time countdown starts again.

## 10.3. MODE 0 PIR Cycle

Condition: night_mode = 1.

A PIR 0→1 edge activates max_bright_bit = 1, PIR_bit = 1 and max_bright_sec = 0.

cnt11 is reset to 0 only if max_bright_bit was previously 0. If the PIR cycle is already active, a new edge does not cause another fade-down/fade-up.

max_bright gradually increases cnt11 to 255 and uses it to drive R/G/B/W as white lighting.

After the PIR output changes to 0, Timer1 starts counting max_bright_sec.

When max_bright_sec_set is reached, max_bright decreases cnt11 to 0; max_bright_bit, PIR_bit and max_bright_sec are then cleared and the MODE 0 RGB background effect returns.

## 10.4. MODE 1/2 PIR Cycle

The PIR edge is used only in darkness and only for the initial activation of PIR mode when manual lighting is not switched on.

If dim_sw = 0, rgb_eff_bit = 0 and PIR_bit = 0, the first PIR edge activates PIR_bit.

Only during this first PIR activation is the dynamic RGB effect prepared to start from the blue phase: cnt11 = 0, phase = 4, cnt = 0, tick = 0, speedSlowCnt = 0. Subsequent PIR edges, while PIR_bit = 1, no longer modify the state of the dynamic effect.

For static effects, dimm_up_dwn performs the gradual transition.

For dynamic effects 0/1, the blue channel first gradually reaches bright_lim, after which RGB_dyn continues the color cycle.

While PIR = 1 and PIR_bit = 1, max_bright_sec is held at 0. When PIR becomes 0, the hold-time countdown begins. If PIR becomes 1 again during this time, only max_bright_sec is reset; the dynamic RGB effect counters are not restarted. When max_bright_sec reaches max_bright_sec_set, PIR_bit is cleared and dimm_up_dwn gradually returns the lighting to the inactive/background state.

## 10.5. Photo Sensor Influence

light_detect uses hysteresis around lux_set: darkness is detected above lux_set + 10, while the light-off filter starts below lux_set - 10.

photo_off_sec counts only when photo_filt_bit = 1; after 20 s, night_mode is cleared.

Therefore, the photo sensor must be optically shielded from the LED strip itself. If LED light reaches the sensor, it may falsely trigger a transition to day mode.

## 10.6. Timer Summary

| What activates it | Timer | What happens at the end |
| --- | --- | --- |
| Button 1 switches on the light in MODE 1/2 (dim_sw = 1) | sec / min / hours | With the current code, the lighting is automatically switched off after ~2 h 31 min. |
| RGBW effect is active (rgb_eff_bit = 1) | sec / min / hours | With the current code, the effect is automatically switched off after ~2 h 31 min. |
| A PIR edge in darkness activates PIR_bit | max_bright_sec | Counts only after the PIR output changes to 0; a new edge resets the time. |
| MODE 0 PIR white lighting | max_bright_sec + cnt11 fade | After 50/120/180/240 s, the white light gradually fades down and the RGB background returns. |
| No activity in SET mode for ~30 s | sett_execute_sec | Automatic exit from SET, MODE indication and beep_set = 20. |

Note: This manual has been updated according to the latest program logic: PIR edge mode, removal of min_bright_bit, new SET 3 times, blue fade-in for the dynamic RGBW effect, and periodic PIR warnings.