GCBASIC I2C Bus Scanner

Software I2C + Software Serial (softserial.h)

Setup, Wiring, Configuration and Usage Guide

# 1. Overview

This program scans the full I2C address space (0x00 to 0xFF) and prints a formatted map of every device that responds to the serial terminal. It uses GCBASIC's software (bit-banged) I2C and the optimised softserial.h library for serial output — no hardware USART is required.

When complete, the terminal will display a 16x16 hex grid. Each cell shows the device address in hex if a device responded there, or '--' if nothing was found.

NOTE: This is a one-shot scan. The program runs once, prints the map, and halts. Reset or re-program the chip to run again.

# 2. What You Need

| Hardware | Description |
| --- | --- |
| Microcontroller | Supported by GCBASIC |
| I2C pull-ups x2 | 4.7 kOhm resistors — one on SDA, one on SCL, both to Vdd |
| I2C device(s) | Any I2C peripheral: OLED, RTC, sensor, EEPROM, etc. |
| USB-Serial adapter | 3.3V FTDI, CP2102, CH340 or similar TTL serial adapter |
| GCBASIC IDE | Installed and configured for your programmer |
| Terminal software | PuTTY, TeraTerm, CoolTerm or Arduino Serial Monitor |

NOTE: The softserial.h library is direct-connect TTL serial. Do NOT use a MAX232 RS-232 level shifter with a USB adapter — connect TTL TX directly to the adapter's RX pin.

# 3. Circuit Preparation and Pull-up Testing

Before loading any software, verify the hardware is correct. Work through these steps in order. Check each one off before moving on.

## 3.1  Power and Pull-up Resistors

- Power OFF the circuit completely.

- Install a 4.7 kOhm resistor between SDA (PORTC.5) and Vdd.

- Install a second 4.7 kOhm resistor between SCL (PORTC.4) and Vdd.

- Power ON the circuit.

- Using a multimeter set to DC volts, measure SDA to GND. You should read close to Vdd (e.g. 3.28V or 4.95V). A reading of 0V means the pull-up resistor is missing or the line is shorted to GND.

- Measure SCL to GND. Same result expected — close to Vdd at idle.

WARNING: If either line reads 0V with no I2C device connected, stop. A missing pull-up causes every address to falsely appear as a device (bus stuck LOW). Fix the pull-ups before continuing.

## 3.2  I2C Device Connection

- Connect your I2C device(s): SDA to PORTC.5, SCL to PORTC.4. Connect device Vdd and GND as required by the device datasheet.

- If the device has address-select pins (A0, A1, A2) set them to a known state — do not leave them floating.

- Power ON. Re-measure SDA and SCL with the device connected. Both should still read close to Vdd at idle. If a line is pulled below 0.5V, a device may be holding the bus low — check your wiring.

TIP: A logic analyser or oscilloscope on SDA/SCL is very useful for diagnosing I2C problems but is not required for this scanner to work.

## 3.3  Serial TX Connection

- Connect PORTC.6 (SER1_TXPIN 6) on the PIC to the RX pin of your USB-serial adapter.

- Connect GND of the PIC board to GND of the USB-serial adapter.

- You do not need to connect the adapter TX to the PIC — this program is transmit-only.

- Plug the USB adapter into your PC. Note the COM port that appears in Device Manager (Windows) or /dev/ttyUSBx (Linux/Mac).

WARNING: Match voltage levels. If your PIC runs at 3.3V use the 3.3V logic setting on your adapter. A 5V signal on a 3.3V IO pin can damage the chip.

# 4. Configuring the Software for Your Hardware

Open the .gcb source file in the GCBASIC IDE. Make the following changes before compiling. Work through them in order.

## 4.1  Select Your Chip

- Find the line:

#chip 16F17556

- Change 16F17556 to the exact part number of your chip. Examples:

- #chip 16F18875

- #chip 18F4550

- #chip 16F877A

- Verify the chip is in GCBASIC's supported device list before compiling.

## 4.2  Set the I2C Pins

- Find these two lines:

#define I2C_DATA  PORTC.5

#define I2C_CLOCK PORTC.4

- Change PORTC.5 to the port and pin you have wired SDA to on your specific chip.

- Change PORTC.4 to the port and pin you have wired SCL to.

NOTE: Format is always PORT<letter>.<bit> for example PORTB.3 or PORTA.0

## 4.3  Set the Serial TX Pin

- Find these two lines:

#define SER1_TXPORT PORTC    ; port letter only

#define SER1_TXPIN  6        ; bit number

- Change SER1_TXPORT to the port letter only (no bit number) for your TX pin. Example: PORTB

- Change SER1_TXPIN to the bit number of your TX pin. Example: 1

NOTE: softserial.h uses a two-part port/pin definition — not the PORTC.6 single-line format used for I2C. Keep them as two separate defines.

## 4.4  Set the Baud Rate

- Find:

#define SER1_BAUD 9600

- Change 9600 to your preferred baud rate. Common values: 9600, 19200, 38400, 57600, 115200.

- Set your terminal software to exactly the same value.

- You terminal should be set to these parameters.

  - The select COM port will be different.

  - Ensure ASCII is selected, not HEX ( shown in red ).

  - Baud rate, 9600 recommended – this MUST match the GCBASIC source setting.

  - 8 bit, parity None, handshaking None

TIP: At slower clock speeds (e.g. 4 MHz internal oscillator) stick to 9600 baud. At 16 MHz or higher you can reliably use 57600 or 115200.

## 4.5  Startup Delay

- Find:

wait 500 ms

- This 500 ms pause lets your terminal connect before output starts. Increase to 1000 ms or 2000 ms if you consistently miss the first lines.

# 5. Compile and Program the Chip

- Save your edited .gcb file.

- In the GCBASIC IDE click Compile (F5). Fix any errors before continuing.

- Connect your programmer (PICkit, SNAP, ICD, etc.) to the chip.

- Click Program to flash the chip. Wait for the programmer to confirm success.

- Disconnect the programmer. The chip will now run the scan automatically on every power-up.

# 6. Using the Program

- Open your terminal software.

- Select the correct COM port for your USB-serial adapter.

- Set baud rate to match SER1_BAUD in the source (default 9600).

- Set: 8 data bits, 1 stop bit, No parity, No flow control.

- Open the connection / click Connect.

- Power ON or reset the PIC. After the startup delay the scan begins automatically — no keypress needed.

- Watch the terminal. The program prints the header row then scans all 256 addresses, printing each row as it completes.

- When 'End of Search' appears the scan is done. The program has halted.

# 7. Understanding the Output

The output is a 16-column hex grid. Each row covers 16 consecutive I2C addresses. The row label on the left is the high byte of the address range.

Software I2C Live Demo

00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F

00: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

10: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

20: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

30: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

40: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

50: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

60: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

70: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

80: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

90: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

A0: A0 A1 -- -- -- -- -- -- -- -- -- -- -- -- -- --

B0: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

C0: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

D0: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

E0: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

F0: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

End of Search

In this example two devices responded: one at 0xA0 and one at 0xA1.

| Symbol | Meaning |
| --- | --- |
| -- | No device responded at this address |
| A0 (hex value) | A device ACK'd — the address is shown in hex |
| Row label A0: | First address of that row (column 00 offset) |
| Header 00..0F | Column offset — add to row label for the full address |

NOTE: I2C standard 7-bit addresses run from 0x00 to 0x7F. This scanner transmits the full 8-bit value (address shifted left with the R/W bit). Divide any displayed address by 2 to get the 7-bit address — or check your device datasheet which often lists the 8-bit write address directly.

# 8. Troubleshooting

## No output appears in the terminal

- Confirm the COM port — it can change each time you plug in the adapter.

- Confirm baud rate in terminal matches SER1_BAUD exactly.

- Confirm SER1_TXPORT and SER1_TXPIN point to the pin you physically wired.

- Measure the TX pin at idle with a multimeter — it should read close to Vdd (idle HIGH for normal TTL). A reading of 0V means the pin is wrong or SER1_INVERT should be On.

- Increase startup delay to 'wait 2000 ms' and reconnect the terminal before resetting the chip.

## Output is garbled or shows wrong characters

- Baud rate mismatch — verify both sides are identical.

- Check SER1_INVERT setting — should be Off for a standard USB-TTL adapter.

- Verify your chip clock speed. If using the internal oscillator confirm it is configured correctly in GCBASIC chip settings or config bits.

## Every address shows a device (entire grid filled)

- Pull-up resistors missing or wrong value. Repeat Section 3.1 tests.

- SDA or SCL shorted to GND. Check wiring carefully.

## No devices found (all '--') but a device is connected

- Re-check I2C_DATA and I2C_CLOCK pin assignments in the source vs your actual wiring.

- Confirm pull-up resistors are connected to Vdd — not GND.

- Verify device Vdd and GND connections.

- Check address-select pins — floating pins give unpredictable addresses.

- Confirm the device operates at your supply voltage.

## Chip does not appear to run after programming

- Verify the correct chip is selected in #chip.

- Confirm programmer reported a successful write with no errors.

- Measure the chip Vdd pin to confirm it is powered.

- Increase startup delay to 'wait 2000 ms' and retry.

# 9. Quick Reference — Defines to Change

| Define | What it controls |
| --- | --- |
| #chip 16F17556 | Your exact PIC part number |
| I2C_DATA PORTC.5 | Port.Pin for I2C SDA |
| I2C_CLOCK PORTC.4 | Port.Pin for I2C SCL |
| SER1_TXPORT PORTC | Port letter only for serial TX |
| SER1_TXPIN 6 | Bit number for serial TX pin |
| SER1_BAUD 9600 | Baud rate — must match your terminal |
| wait 500 ms | Startup delay before scan begins |

# 10. Full Annotated Source Listing

The complete program with inline comments. This is the file you edit and compile.

'''============================================================

''' GCBASIC I2C Bus Scanner

''' Uses: Software I2C + softserial.h (bit-banged serial TX)

''' Target: PIC 16F17556 -- change #chip for your device

''' Author: EvanV Licence: GPL Version: 1.1a

'''============================================================

'''

''' EXAMPLE OUTPUT (two devices found at 0xA0 and 0xA1):

'''

''' Software I2C Live Demo

'''

''' 00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F

''' 00: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

''' 10: -- (rows continue) --

''' A0: A0 A1 -- -- -- -- -- -- -- -- -- -- -- -- -- --

''' F0: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

'''

''' End of Search

'''

''' WIRING (defaults -- change defines below):

''' SDA -> PORTC.5 (with 4.7k pull-up to Vdd)

''' SCL -> PORTC.4 (with 4.7k pull-up to Vdd)

''' TX -> PORTC.6 pin (to RX of USB-serial adapter)

''' GND -> GND of USB-serial adapter

'''============================================================

; ----- STEP 1: Set your chip part number

#chip 16F17556

#option explicit ; require all variables to be declared

; ----- STEP 2: Include optimised software serial library

#include <softserial.h>

; ----- STEP 3: I2C pin configuration

; ----- CHANGE PORTC.5 and PORTC.4 to your wired pins

#define I2C_MODE Master

#define I2C_DATA PORTC.5 ; SDA

#define I2C_CLOCK PORTC.4 ; SCL

#define I2C_DISABLE_INTERRUPTS ON

; ----- STEP 4: Serial TX configuration

; ----- CHANGE SER1_TXPORT and SER1_TXPIN to your wired pin

#define SER1_BAUD 9600 ; match your terminal baud rate

#define SER1_DATABITS 8 ; 8 data bits

#define SER1_STOPBITS 1 ; 1 stop bit

#define SER1_INVERT Off ; Off = idle HIGH (normal TTL / USB adapter)

#define SER1_TXPORT PORTC ; port letter only (no .bit)

#define SER1_TXPIN 6 ; bit number of TX pin

; ----- STEP 5: Startup delay

; ----- Increase to 2000 ms if terminal misses the first lines

wait 500 ms

; ----- Variables

Dim DeviceID as Byte

Dim DisplayNewLine as Byte

; ----- SendCRLF subroutine

; ----- softserial.h uses Ser1Send for raw byte output.

; ----- ASCII 13 = Carriage Return (CR)

; ----- ASCII 10 = Line Feed (LF)

Sub SendCRLF

Ser1Send 13 ' CR

Ser1Send 10 ' LF

End Sub

; ====== MAIN PROGRAM =========================================

SendCRLF

Ser1Print "Software I2C Live Demo"

SendCRLF

SendCRLF

; ----- Print column header: 00 01 02 03 ... 0F

Ser1Print " "

for DeviceID = 0 to 15

Ser1Print hex(DeviceID)

Ser1Send 32 ' ASCII 32 = space

next

; ----- Scan all 256 I2C addresses

for DeviceID = 0 to 255

DisplayNewLine = DeviceID % 16

if DisplayNewLine = 0 Then

SendCRLF

Ser1Print hex(DeviceID) ' row label e.g. 'A0:'

Ser1Print ": "

end if

Ser1Send 32 ' space before each cell

I2CStart

I2CSend ( DeviceID ) ' send address byte

if I2CSendState = True then ' True = device ACK'd (found)

Ser1Print hex(DeviceID) ' print the address

else

Ser1Print "--" ' no device at this address

end if

I2CSend ( 0 ) ' dummy byte before stop condition

I2CStop

Next

SendCRLF

SendCRLF

Ser1Print "End of Search"

SendCRLF

SendCRLF

END